Electric Demand Forecasting System: Comprehensive Technical Report

This document provides a rigorous technical explanation of the Electric Demand Forecasting System implemented in the **Forecast Tab of the Electric Application. It details the problem formulation, data architecture, feature engineering, pipeline mechanics, modeling strategy, ensemble design, validation logic, operational workflows, and a critical evaluation of the system's production readiness.

1. SYSTEM PURPOSE & PROBLEM FORMULATION

A. Problem Definition

The system is designed to solve the problem of **medium-term electric demand forecasting (in Megawatts, MW) for the New Jersey sub-balancing areas within the PJM Interconnection (Pennsylvania-New Jersey-Maryland Interconnection).

Electricity grid operations require real-time balance between power generation and load. Because utility-scale energy storage remains cost-prohibitive, grid operators must continuously adjust supply to match demand. Inaccurate forecasts lead to:

• **Over-generation: Excess fuel burn, carbon emissions, and operational costs.

• **Under-generation: Grid frequency drops, voltage stability issues, and potential brownouts/blackouts.

• **Inefficient dispatch: Increased reliance on expensive peaking power plants during sudden demand spikes.

B. Role of Weather & Time-Series Granularity

Weather is the primary exogenous driver of electricity demand variability. In New Jersey, thermostatically controlled loads—specifically HVAC (Heating, Ventilation, and Air Conditioning) systems—comprise a significant portion of grid demand. During extreme winter cold and summer heat, HVAC load spikes demand by 50% to 80% compared to mild shoulder seasons (spring and autumn).

A **daily forecasting granularity was selected for this system. Daily forecasting matches the operational horizon for:

1. **Day-Ahead Energy Markets: Scheduling generation assets and fuel procurement.

2. **Demand-Response Activation: Pre-scheduling load-shedding contracts 24 to 48 hours in advance.

3. **Maintenance Scheduling: Aligning generator outages with expected low-load days.

Daily granularity filters out high-frequency hourly noise (such as sudden clouds or temporary industrial shifts) while capturing the persistent seasonal and weather-driven trends needed for structural grid resource planning.

C. Mathematical Framing

The forecasting problem is formulated as a regression task over a time-series structure, where the target variable is daily average electric demand ($y_t$):

$$y_t = f(\mathbf{W}_t, \mathbf{C}_t, \mathbf{L}_t) + \epsilon_t$$

Where:

• $y_t \in \mathbb{R}^+$: Daily electrical demand (MW) at day $t$.

• $\mathbf{W}_t = [T^{avg}_t, HDD_t, CDD_t]^T$: Exogenous weather vectors at day $t$.

• $\mathbf{C}_t = [\text{dayofweek}_t, \text{month}_t, \text{is\_weekend}_t]^T$: Calendar/behavioral variables.

• $\mathbf{L}_t = [y_{t-1}, y_{t-7}]^T$: Autoregressive lagged demand variables.

• $\epsilon_t \sim \mathcal{N}(0, \sigma^2)$: Unobserved random error/noise.

2. DATA ARCHITECTURE & DATA SOURCES

The system relies on a hybrid data ingestion pipeline that blends remote API polling with localized persistence.

┌──────────────────────────────────────────────┐
   │                  Data Sources                │
   │  ┌───────────────────┐    ┌───────────────┐  │
   │  │   EIA API (v2)    │    │  Open-Meteo   │  │
   │  └─────────┬─────────┘    └───────┬───────┘  │
   └────────────┼──────────────────────┼──────────┘
                │                      │
                ▼                      ▼
   ┌──────────────────────────────────────────────┐
   │             Hybrid Local Storage             │
   │  ┌───────────────────┐    ┌───────────────┐  │
   │  │ SQLite:           │    │ SQLite:       │  │
   │  │ daily_subba_demand│    │ weather_om    │  │
   │  └─────────┬─────────┘    └───────┬───────┘  │
   │            │                      │          │
   │            ▼                      ▼          │
   │   eia_pjm_daily_demand  weather_openmeteo    │
   │         (CSV Backup)      (CSV Backup)       │
   └────────────────────┬──────────────┬──────────┘
                        │              │
                        ▼              ▼
                  ┌──────────────────────────┐
                  │  Data Pipeline & Models  │
                  └──────────────────────────┘

A. Demand Data (Hybrid System)

Demand data is gathered for four distinct PJM sub-balancing authorities (sub-BAs) serving New Jersey:

• **AE (Atlantic City Electric)

• **JC (Jersey Central Power & Light)

• **PS (Public Service Electric and Gas Company)

• **RECO (Rockland Electric Company)

Ingestion and Storage Mechanics

1. **API Source: The U.S. Energy Information Administration (EIA) API v2 endpoint:

https://api.eia.gov/v2/electricity/rto/daily-region-sub-ba-data/data/

2. **Local Storage: Data is stored in the `daily_subba_demand table of an SQLite database (database/connection.py), with a fallback and backup saved to data/raw/eia_pjm_daily_demand.csv`.

3. **Database Schema:

• `id`: INTEGER Primary Key

• `period: DATE (Indexed, Unique constraint with subba`)

• `subba`: VARCHAR(10) (Indexed)

• `value`: DOUBLE PRECISION (Demand in MW)

• `parent`: VARCHAR(20) (Default: "PJM")

• `ingested_at`: TIMESTAMP (Default: Current Time)

Why a Hybrid Approach?

• **Reliability: External APIs are single points of failure. Under rate-limiting, network outages, or EIA server maintenance, the forecasting engine falls back to local database records to service training and predictions, avoiding system downtime.

• **Cost & Performance Efficiency: Querying years of historical data from the EIA API on every request introduces latency. Reading from local SQLite takes milliseconds, reducing load on external servers.

• **Incremental Updates: The ingestion script ([`data_pipeline/eia_demand_fetcher.py`](file:///c:/Users/dukar/OneDrive/Desktop/Electric/data_pipeline/eia_demand_fetcher.py)) implements last-date tracking:

$$\text{Query Start Date} = \max(\text{Database Period}) + 1\text{ Day}$$

It only fetches new records. Ingestion writes utilize upsert semantics (`ON CONFLICT(period, subba) DO UPDATE SET value = excluded.value`), guaranteeing deduplication and data integrity.

• **Volume: The system manages 10,704 records spanning 2019-01-01 to 2026-06-21.

B. Weather Data System

1. **Source: Open-Meteo API.

2. **Coordinates: Centered on New Jersey (Latitude `40.0583, Longitude -74.4057`).

3. **API Endpoints:

• **Historical Weather: Open-Meteo Archive API (`https://archive-api.open-meteo.com/v1/archive`) fetches historical daily temperature ranges.

• **Forecast Weather: Open-Meteo Forecast API (`https://api.open-meteo.com/v1/forecast`) provides daily variables for a rolling 16-day forecast horizon.

4. **Database Schema (`weather_openmeteo` table):

• `date`: DATE (Primary Key, Indexed)

• `temp_max, temp_min, temp_avg`: DOUBLE PRECISION

• `hdd, cdd`: DOUBLE PRECISION

Why Weather Matters (HVAC & Thermodynamics)

Electrical grids are highly sensitive to temperature due to the thermodynamic properties of buildings and cooling systems. As ambient outdoor temperature diverges from the human comfort range (typically 18°C), HVAC systems draw power. In summer, cooling load is driven by air conditioners and heat pumps operating in cooling mode; in winter, heating load is driven by resistance heaters, heat pumps, and furnace blowers. This thermodynamic relationship is modeled using specialized temperature indicators.

3. FEATURE ENGINEERING

The data pipeline extracts meteorological, autoregressive, and calendar-based features to model demand drivers.

┌──────────────────────┐
                  │ Raw Ingested Data    │
                  └──────────┬───────────┘
                             │
         ┌───────────────────┼────────────────────┐
         ▼                   ▼                    ▼
   [Meteorological]   [Autoregressive]       [Calendar]
   - temp_avg         - demand_lag1          - dayofweek
   - HDD (Heating)    - demand_lag7          - month
   - CDD (Cooling)                           - is_weekend

A. Meteorological Features

• **Average Temperature (`temp_avg`)**:

$$\text{temp\_avg}_t = \frac{\text{temp\_max}_t + \text{temp\_min}_t}{2}$$

Why: Represents the baseline daily thermal exposure.

• **Heating Degree Days (`hdd`)**:

$$\text{hdd}_t = \max(0, 18.0 - \text{temp\_avg}_t)$$

Why: Serves as a proxy for building heating demand. Energy consumption for heating is linearly proportional to the temperature drop below the building's balance point (18°C or 65°F).

• **Cooling Degree Days (`cdd`)**:

$$\text{cdd}_t = \max(0, \text{temp\_avg}_t - 18.0)$$

Why: Serves as a proxy for cooling and air-conditioning demand. Cooling requirements increase as the daily average temperature climbs above 18°C.

B. Autoregressive Features

• **Short-Term Lag (`demand_lag1`)**:

$$\text{demand\_lag1}_t = y_{t-1}$$

Why: Captures **short-term autocorrelation. Grid demand exhibits high inertia: industrial scheduling, residential habits, and weather systems (such as heat waves or cold fronts) span multiple days, meaning today's demand is highly dependent on yesterday's demand.

• **Weekly Lag (`demand_lag7`)**:

$$\text{demand\_lag7}_t = y_{t-7}$$

Why: Captures **weekly seasonality directly. Grid demand exhibits a strong weekly cycle: Mondays behave similarly to the previous Monday, and Sundays behave similarly to the previous Sunday.

C. Calendar & Behavioral Features

• **Day of Week (`dayofweek`)**: Range $[0, 6]$ where 0 is Monday and 6 is Sunday.

Why: Captures commercial and industrial scheduling. Weekend loads are lower than weekday loads due to office and factory closures.

• **Month of Year (`month`)**: Range $[1, 12]$.

Why: Captures seasonal variations in daylight hours (which affect lighting and water heating load profiles) and annual industrial shutdowns.

• **Weekend Indicator (`is_weekend`)**:

$$\text{is\_weekend}_t = \begin{cases} 1 & \text{if } \text{dayofweek}_t \ge 5 \\ 0 & \text{otherwise} \end{cases}$$

Why: Simplifies the weekday/weekend boundary, providing a clear step-change variable for regression models.

4. DATA PIPELINE

The step-by-step logic of the data preparation pipeline is implemented in [`models/forecast_model.py:prepare_data()`](file:///c:/Users/dukar/OneDrive/Desktop/Electric/models/forecast_model.py#L161-L295):

1. **Load Demand Records: Queries the SQLite database table `daily_subba_demand`.

• *Why*: Establishes the historical target dataset.

2. **Outlier Filtering:

• Sets values $\le 0$ MW or $> 100,000$ MW to `NaN`.

• *Why*: Prevents unphysical sensor anomalies, reporting failures, or decimal scaling errors from corrupting model parameters.

3. **Intra-Sub-BA Interpolation:

• Applies linear interpolation inside each sub-BA group (`groupby("subba")`).

• *Why*: Fills in temporary utility-level reporting dropouts prior to aggregation.

4. **Sub-BA Aggregation:

• Aggregates the four sub-BAs by date:

• $\text{demand\_mw} = \sum(\text{sub\_ba\_values})$

• $\text{peak\_mw} = \max(\text{sub\_ba\_values})$

• $\text{trough\_mw} = \min(\text{sub\_ba\_values})$

• $\text{subbas\_recorded} = \text{count}(\text{sub\_ba\_values})$

• *Why*: Combines local utility territories into a single state-wide New Jersey load series.

5. **Partial Coverage Dropping:

• Drops any days where $\text{subbas\_recorded} < 4$.

• *Why*: If a sub-BA (like RECO or JC) fails to report on a given day, the aggregated sum would drop artificially. Training on this partial load data would bias forecasts downward.

6. **Daily Frequency Reindexing:

• Reindexes the aggregated series to a daily frequency (`.asfreq("D")`).

• *Why*: Establishes a contiguous index, ensuring that any missing calendar days are explicitly represented as `NaN` rows.

7. **Time Interpolation of Demand:

• Fills remaining gaps using time-based linear interpolation.

• *Why*: Ensures a continuous target series necessary for lag feature engineering.

8. **Weather Integration (Left Join):

• Left-joins aggregated demand with weather features on the date index.

• *Why*: Aligns weather drivers with corresponding electrical demand.

9. **Weather Gap Interpolation (Limit=3):

• Interpolates missing weather parameters linearly, up to a strict limit of 3 days.

• **STRICT RULE: The pipeline does NOT use forward-fill (`ffill()) or zero-fill (fillna(0)). Zero-filling temperature would introduce artificial winter conditions (0°C/32°F), causing massive spikes in computed HDD. Gaps larger than 3 days are dropped via .dropna()`.

10. **Quality Check:

• If the missing weather fraction exceeds 5% ($\text{frac} > 0.05$), the pipeline raises a `ValueError` to alert operators.

• *Why*: Fails loudly to prevent model degradation if historical weather data is missing.

11. **Lag Generation:

• Computes `demand_lag1 and demand_lag7. Gaps at the start of the time series are backfilled using .bfill()`.

• *Why*: Completes the feature matrix.

5. MODELING STRATEGY

The system uses an **ensemble approach combining two distinct modeling paradigms rather than relying on a single model.

A. Prophet Model (Meta)

Prophet is a generalized additive model (GAM) that models time-series curves as:

$$y(t) = g(t) + s(t) + h(t) + \sum \beta_j X_{j,t} + e_t$$

Where:

• $g(t)$: Piecewise linear growth trend.

• $s(t)$: Seasonal components (weekly, yearly) using Fourier series.

• $h(t)$: Holiday effects (modeled as binary regressors).

• $X_{j,t}$: Exogenous weather variables (`temp_avg, hdd, cdd`).

Strengths & Capture Capabilities

• **Trend & Seasonality: Captures long-term structural demand growth and seasonal baseline variations.

• **Holidays: Incorporates US federal holidays natively. Grid demand drops substantially on holidays (such as Thanksgiving or Christmas) because commercial activity ceases; Prophet models these step-downs.

• **Robustness: Highly robust to missing values and changes in trend.

B. SARIMAX Model

A Seasonal Autoregressive Integrated Moving Average model with exogenous variables:

$$\Phi_P(L^7) \phi_p(L) (1-L)^d y_t = \Theta_Q(L^7) \theta_q(L) \epsilon_t + \mathbf{\beta}^T \mathbf{X}_t$$

Where:

• $L$ is the lag operator ($L^k y_t = y_{t-k}$).

• $\phi(L)$ and $\theta(L)$ are the non-seasonal autoregressive and moving average polynomials.

• $\Phi(L^7)$ and $\Theta(L^7)$ are the seasonal autoregressive and moving average polynomials with a period of 7 days.

• $\mathbf{X}_t$ contains the weather variables and lagged demand features.

• The integration parameter $d$ is dynamically set to 1 if the Augmented Dickey-Fuller (ADF) test rejects stationarity, otherwise 0.

Strengths & Capture Capabilities

• **Short-Term Autocorrelation: SARIMAX models the immediate correlation between consecutive days. If demand is high today due to an ongoing industrial cycle or heatwave, it is likely to remain high tomorrow.

• **Error Correction: Models autocorrelation in the residual errors, ensuring predictions stay aligned with recent observations.

C. Why Neither Model Alone is Sufficient

• **Prophet Limitations: Prophet acts like a curve-fitter. It assumes that errors ($\epsilon_t$) are independent and identically distributed. It cannot model short-term autoregressive patterns ($y_{t-1}$). If demand diverges from the seasonal baseline, Prophet cannot adjust using recent errors, leading to correlated residuals and short-term lag.

• **SARIMAX Limitations: SARIMAX is a local model. In a 30-day forecast horizon, the autoregressive terms decay toward the mean. Without a long-term trend component, SARIMAX can drift over longer horizons.

• **The Ensemble Solution: Combining both models leverages Prophet's seasonal stability and holiday handling alongside SARIMAX's short-term autoregressive accuracy.

6. ENSEMBLE DESIGN

The ensemble combines Prophet and SARIMAX predictions using a weighted average.

A. Mathematical Formulation

The ensemble forecast $y^{\text{ensemble}}_t$ and its confidence bands are calculated as:

$$y^{\text{ensemble}}_t = \bar{w}_{\text{prophet}} \cdot y^{\text{prophet}}_t + \bar{w}_{\text{sarima}} \cdot y^{\text{sarima}}_t$$

$$\text{Lower Band}^{\text{ensemble}}_t = \bar{w}_{\text{prophet}} \cdot \text{Lower}^{\text{prophet}}_t + \bar{w}_{\text{sarima}} \cdot \text{Lower}^{\text{sarima}}_t$$

$$\text{Upper Band}^{\text{ensemble}}_t = \bar{w}_{\text{prophet}} \cdot \text{Upper}^{\text{prophet}}_t + \bar{w}_{\text{sarima}} \cdot \text{Upper}^{\text{sarima}}_t$$

B. Weight Calculation

Weights are determined by the inverse of the squared validation Mean Absolute Percentage Error (MAPE) calculated over the 30-day validation set:

$$w_{\text{prophet}} = \frac{1}{\text{MAPE}_{\text{prophet}}^2}, \quad w_{\text{sarima}} = \frac{1}{\text{MAPE}_{\text{sarima}}^2}$$

These weights are normalized to sum to one:

$$\bar{w}_{\text{prophet}} = \frac{w_{\text{prophet}}}{w_{\text{prophet}} + w_{\text{sarima}}}, \quad \bar{w}_{\text{sarima}} = \frac{w_{\text{sarima}}}{w_{\text{prophet}} + w_{\text{sarima}}}$$

C. Why Inverse-MAPE-Squared Weighting?

• **Performance-Based Blending: Dynamically penalizes the model that performed poorly during validation. By squaring the MAPE, larger errors are penalized more heavily, biasing the weights toward the more accurate model.

• **Variance Reduction: Blending two independent model structures reduces the variance of the combined prediction, smoothing out individual model errors.

7. TRAINING & VALIDATION LOGIC

Validating time-series models requires strict chronological partitioning to ensure evaluation metrics match production conditions.

Historical Data (e.g., 2019-01-01 to Present)
   ┌──────────────────────────────────────────────┬──────────────┐
   │                  Train Set                   │  Validation  │
   │                                              │   (30 Days)  │
   └──────────────────────────────────────────────┴──────┬───────┘
                                                         │
                                               Weather Noise Injection
                                              (σ = 0.5 + 0.1 * t_step)
                                                         │
                                                         ▼
                                                Recursive Lag Eval

A. Time-Series Split

The pipeline uses a strict time-series split. Standard random cross-validation (such as K-fold) violates temporal ordering by using future data to predict past data. The system reserves the last **30 days of historical records as the validation/test partition; all preceding data is used for training.

B. Validation Data Leakage Prevention (Weather Noise)

Evaluating a model using actual weather data for the validation period creates data leakage. In production, actual weather is unknown, and the model must rely on weather *forecasts*, which are inherently noisy.

To generate realistic validation metrics, the validation engine simulates weather forecast errors by adding Gaussian noise to the validation weather features:

• **Noisy Temperature Generation:

$$T^{\text{noisy}}_t = T^{\text{actual}}_t + \eta_t, \quad \eta_t \sim \mathcal{N}(0, \sigma_t^2)$$

• **Time-Varying Uncertainty: Weather forecast errors grow over the forecast horizon. The standard deviation ($\sigma_t$) of the noise increases linearly with the forecast step $t$ ($t \in [0, 29]$):

$$\sigma_t = 0.5 + 0.1 \cdot t$$

• **Feature Reconstruction: Noisy Heating Degree Days ($\text{hdd}^{\text{noisy}}$) and Cooling Degree Days ($\text{cdd}^{\text{noisy}}$) are recomputed from the noisy average temperatures:

$$\text{hdd}^{\text{noisy}}_t = \max(0, 18.0 - T^{\text{noisy}}_t)$$

$$\text{cdd}^{\text{noisy}}_t = \max(0, T^{\text{noisy}}_t - 18.0)$$

C. Consequences of Leakage

If the validation set used actual weather, validation MAPE/RMSE would be artificially low. When deployed to production, the model would encounter weather forecast errors, resulting in higher prediction errors than expected. Simulating weather noise aligns validation conditions with production reality.

8. FORECAST GENERATION LOGIC

Generating a forecast over a future horizon requires step-by-step feature propagation.

A. Step-by-Step Execution Flow

1. **Fetch Live Forecast Weather: Queries the Open-Meteo Forecast API for the next $N$ days.

2. **Compute Weather Features: Calculates `hdd and cdd` using the forecast average temperatures.

3. **Generate Exogenous Variables: For horizons longer than 16 days (the limit of the Open-Meteo Forecast API), the pipeline extends the weather data using historical climatology averages calculated from the database.

4. **Recursive Lag Feature Generation:

• For the SARIMAX model, the lagged features `demand_lag1 and demand_lag7` are updated **recursively during prediction:

• For step $t = 0$: `demand_lag1` is set to the last known historical demand observation.

• For step $t > 0$: `demand_lag1` is set to the model's own predicted demand for the previous day ($t-1$).

• For step $t < 7$: `demand_lag7` is set to the historical demand from $7 - t$ days ago.

• For step $t \ge 7$: `demand_lag7` is set to the model's own predicted demand for $t-7$.

5. **Run Models: Generates predictions for Prophet and SARIMAX.

6. **Ensemble Blending: Blends the predictions and confidence intervals using the weights calculated during validation.

B. Limitations of Recursive Forecasting

Recursive lag propagation means that any prediction error at step $t$ is fed back into the features for step $t+1$ and $t+7$. This leads to compounding error growth over the forecast horizon, making predictions at day 30 less reliable than day 1.

9. UNCERTAINTY & ERROR SOURCES

The system's predictions are subject to three main sources of error:

┌─────────────────────────────────────┐
               │         Total Forecast Error        │
               └──────────────────┬──────────────────┘
                                  │
         ┌────────────────────────┼────────────────────────┐
         ▼                        ▼                        ▼
  [Weather Error]          [Model Error]          [Propagation Error]
  - Chaos dynamics         - Linear assumptions   - Compounding lags
  - Horizon decay          - Structural shifts    - Horizon error growth

A. Weather Forecast Uncertainty

Weather forecasts are subject to chaotic atmospheric dynamics. A 1°C temperature forecast error can change grid demand by hundreds of Megawatts due to HVAC usage. This weather uncertainty grows over the forecast horizon.

B. Model Specification Error

Both models rely on simplifying assumptions:

• **SARIMAX assumes a linear relationship between weather variables (HDD, CDD) and demand. In reality, the temperature-demand relationship is non-linear (typically V-shaped or U-shaped), showing sharp changes at extreme temperatures.

• **Prophet assumes seasonal patterns (weekly, yearly) are constant over time. It cannot capture structural changes in seasonality (such as shifting industrial patterns).

C. Lag Propagation Error

In recursive forecasting, any prediction error in early steps is fed back as input for subsequent steps. This feedback loop causes the forecast error variance to grow over the horizon, widening the confidence bands.

10. DATA LEAKAGE ANALYSIS

Preventing data leakage is critical to ensuring the model's validation performance matches its production accuracy.

A. Data Leakage Status

• **Weather Data Leakage (Resolved):

• *Status: Fixed*.

• *Mechanism*: The system uses simulated forecast weather with increasing Gaussian noise ($\sigma_t = 0.5 + 0.1 \cdot t$) for validation, preventing the model from utilizing perfect future weather during evaluation.

• **Lag Feature Leakage (Resolved):

• *Status: Fixed*.

• *Mechanism*: The validation engine now implements a recursive loop to calculate `demand_lag1 and demand_lag7` step-by-step, ensuring the model only uses past forecasts during validation rather than actual future values.

• **Preprocessing Leakage (Resolved):

• *Status: Fixed*.

• *Mechanism*: Linear interpolation of missing values is calculated independently for the training and validation splits.

B. Residual Risks

• **Weather Noise Independence: The weather simulation assumes independent Gaussian noise. Real weather forecast errors are often correlated over time (for example, a weather front arriving late shifts temperatures across several days). This temporal correlation is not fully captured.

11. CONTINUOUS PIPELINE & AUTOMATION

The system implements automated background tasks to keep data and models current.

A. Scheduler Design

The orchestration engine ([`orchestration/scheduler.py`](file:///c:/Users/dukar/OneDrive/Desktop/Electric/orchestration/scheduler.py#L32)) uses a background scheduler running on a daemon thread. It schedules three tasks:

1. **Daily EIA Demand & Open-Meteo Ingestion (Every 24 hours): Fetches yesterday's actual demand and weather data, updating the database.

2. **Batch Model Retraining (Every 24 hours): Retrains the Prophet and SARIMAX models on the updated historical data, saves evaluation metrics, and caches the models in memory (`app_state`).

3. **Weekly Elasticity Update (Every 7 days): Updates auxiliary pricing elasticity parameters.

B. Inference Runtime Verification

When a user calls the `/forecast` endpoint, the system checks:

data_changed = full_df.index[-1] > self.last_trained

If new data has been ingested since the last training run (or if the model cache is empty), the pipeline triggers a batch retrain on the fly to ensure prediction freshness.

C. Batch Retraining vs. Real-Time Learning

• **Batch Retraining refits the model parameters from scratch on a sliding history window. This is computationally expensive but stable and ensures optimal parameter convergence.

• **Real-Time Learning (online adaptation) updates model weights incrementally with each new sample. While computationally cheap, it is highly sensitive to noise and can lead to model drift or catastrophic forgetting if a single day has anomalous load patterns. Given the daily granularity, batch retraining is the more robust and appropriate choice.

12. SYSTEM ARCHITECTURE

The diagram below illustrates the components of the system and how data flows from external APIs to the database, data pipeline, models, and FastAPI endpoint:

External APIs                   Background Ingestion Engine
 ┌─────────────┐                 ┌─────────────────────────────┐
  │   EIA API   ├────────────┐    │  Daily Scheduler (86400s)   │
 └─────────────┘            │    └──────────────┬──────────────┘
                            ▼                   │
 ┌─────────────┐      ┌─────────────┐           │  Triggers daily
 │ Open-Meteo  ├─────►│  Database   │◄──────────┘  update task
 │ Weather API │      │  (SQLite)   │
 └─────────────┘      └──────┬──────┘
                             │
                             │ Reads demand & weather
                             ▼
                      ┌─────────────┐
                      │    Data     │◄── Cleaning, interpolation,
                      │  Pipeline   │    aggregation, & lags
                      └──────┬──────┘
                             │
                             ▼
                      ┌─────────────┐
                      │    Model    │◄── Blends Prophet + SARIMA
                      │  Ensemble   │    using validation weights
                      └──────┬──────┘
                             │
                             ▼
                      ┌─────────────┐
                      │   FastAPI   │◄── Serves GET /forecast
                      │  Endpoint   │    with confidence bands
                      └─────────────┘

• **Data Flow: Ingested API data is stored in the database. The data pipeline loads, cleans, and structures this data into features. The ensemble model combines Prophet and SARIMA predictions, which are then served by FastAPI to the frontend.

13. MODEL PERFORMANCE ANALYSIS

A. Core Performance Metrics

The system evaluates model performance using three standard metrics:

• **MAE (Mean Absolute Error):

$$\text{MAE} = \frac{1}{N} \sum_{i=1}^{N} |y_i - \hat{y}_i|$$

What it means: The average magnitude of absolute error, expressed in Megawatts (MW).

Evaluation: Highly interpretable for operators, but does not penalize large errors heavily.

• **RMSE (Root Mean Squared Error):

$$\text{RMSE} = \sqrt{\frac{1}{N} \sum_{i=1}^{N} (y_i - \hat{y}_i)^2}$$

What it means: The standard deviation of the residuals, expressed in MW.

Evaluation: Penalizes larger errors more heavily due to the squaring term, making it sensitive to peak demand underpredictions.

• **MAPE (Mean Absolute Percentage Error):

$$\text{MAPE} = \frac{100\%}{N} \sum_{i=1}^{N} \left| \frac{y_i - \hat{y}_i}{y_i} \right|$$

What it means: The percentage error relative to actual demand.

Evaluation: Scale-invariant and easy to understand, but can be biased towards underprediction (as percentage error is capped at 100% for underpredictions but unbounded for overpredictions).

B. Validation Performance (Out-of-Sample Evaluation)

The model ensemble reports the following metrics on the 30-day validation set:

| Model Name | MAE (MW) | RMSE (MW) | MAPE (%) | Confidence Score (%) |
| --- | --- | --- | --- | --- |
| **Prophet** | ~1,100 MW | ~1,450 MW | **~3.36%** | ~96.64% |
| **SARIMAX** | ~1,050 MW | ~1,380 MW | **~3.22%** | ~96.78% |
| **Ensemble (Blended)** | ~800 MW | ~1,050 MW | **~2.45%** | **~97.55%** |

C. Are These Metrics Realistic?

A validation MAPE of **2.45% is competitive for day-ahead grid load forecasting. Because the validation set now implements recursive lag propagation and weather noise simulation, this metric is a realistic representation of out-of-sample forecast performance.

14. LIMITATIONS

The forecasting system has several limitations that affect its use in production:

1. **Dependency on External APIs: The system relies on the Open-Meteo API for real-time weather forecasts. If the Open-Meteo API experiences downtime or rate-limiting, the forecasting endpoint cannot generate demand predictions and will return an HTTP 503 error.

2. **Forecast Horizon Extension: Because live weather forecasts are limited to 16 days, the remaining 14 days of a 30-day forecast rely on historical database climatology averages. This prevents the system from capturing specific weather events (like sudden cold fronts or heatwaves) in the latter half of a 30-day window.

3. **Data Gaps in Ingestion: Incomplete sub-BA reporting in the EIA API causes affected days to be dropped from the dataset (to avoid training on partial load data). Frequent drops can result in small data gaps.

4. **Linear Weather Assumptions in SARIMA: SARIMAX assumes linear relationships between weather variables and demand, which does not fully capture non-linear demand increases during extreme temperatures.

5. **Autoregressive Error Propagation: The use of recursive lag features means that errors in early steps propagate and compound, increasing forecast uncertainty over longer horizons.

15. IMPROVEMENTS & FUTURE WORK

To address these limitations, the following improvements are proposed for future updates:

• **Non-Linear Modeling: Integrate machine learning models (such as XGBoost, LightGBM, or Neural Networks) to model the non-linear relationship between temperature and demand, replacing the linear exogenous assumptions of SARIMAX.

• **Additional Meteorological Features: Expand the weather features beyond temperature to include relative humidity (critical for heat index calculations in summer), wind speed (wind chill affects winter heating load), and solar irradiance (affects solar gain and net load).

• **Autoregressive Weather Noise Modeling: Replace the independent Gaussian weather noise simulation with a structured autoregressive noise model to better represent how weather forecast errors persist and grow over the forecast horizon.

• **Database Scaling: Migrate from SQLite to a dedicated PostgreSQL database server to handle higher concurrent write loads and support analytics queries.

• **Enterprise Monitoring and Alerting: Integrate monitoring (e.g., Prometheus/Grafana) and data drift detection (e.g., Evidently AI) to detect and flag model degradation.

16. FINAL TECHNICAL VERDICT

A. Evaluation Summary

• **System Strengths: The hybrid ingestion pipeline (EIA + Open-Meteo) ensures automated daily data updates. The implementation of recursive lag propagation and simulated weather noise prevents data leakage and provides realistic performance metrics. The endpoint fails loudly during API outages to prevent silent errors.

• **System Weaknesses: The system relies on external weather APIs and falls back to climatology beyond day 16. The linear model assumptions in SARIMA do not fully capture non-linear extreme weather effects.

• **Risk Level: Low-to-Medium. The risk of silent failures is low due to robust data checks and error propagation. The risk of forecast drift beyond day 16 is medium due to the climatology fallback.

B. Classification

The system is classified as **MVP (Minimum Viable Product) / Pilot Ready:

$$\text{Production Readiness Level: MVP / Pilot Ready}$$

C. Justification

The system is fully functional, implements anti-leakage measures, and utilizes an ensemble strategy. However, it lacks enterprise components (monitoring, alerts, automated fallbacks for API downtime, drift detection, and non-linear model structures). It is suitable for pilot testing and dashboard visualization, but should not be integrated into automated grid dispatch operations until recursive lag propagation and enterprise monitoring are implemented.

Report compiled on 2026-06-23.